LED FLOODLIGHT

INSTRUCTION MANUAL

Specifications and Technical Data：

Luminaire Category:               LED Floodlight

Input voltage:                     AC220-240V  50HZ/60HZ

Ambient Temperature:              -40℃<TA<50℃

WARNING:

1. ENSURE ELECTRICITY IS SWITCHED OFF BEFORE YOU INITIATE ANY MANIPULATIONS.

2. Please read carefully the following instructions before commencing installation.

3. Some of the necessary accessories and spare parts may not be provided including junction box .

4. The Floodlight must be earthed.

5. If the safety shield is damaged, you must replace it immediately, with tempered glass.

6. This is a professional-use product, any installation and/or service must be undertaken by qualified person. Serious injury or death may occur if otherwise.

MOUNTING

- Do not install this product to an unstable surface or any such vibrating one.

- Remove the screw intended for mounting the bracket of the fixture. Remove the bracket.(see fig.2)

- Mark the hole position for fixing bracket onto the wall. Drill the wall at the marked positions (screw not supplied). (see fig.3)

- Screw the fixing bracket to the wall with suitable screw (see fig.4)

- Mount the bracket back to the fixture. Adjust the floodlight to desired position and tighten the screw of fixing bracket. Connect the  mains cable (H05RN-F3G1.0 mm²recommended) with the input lead of our product through the waterproof junction box. Note the Line (-) of the lead. (see fig.5) (Junction box must be IP65 rated or above, ensure to tighten the waterproof screw)

- Turn on the power to make sure the product operates normally. Adjust the irradiation angle according to the intended condition of usage. (see fig.6)

YOU MUST COMPLY strictly to this instructions, otherwise serious injury or death may occur.

DO NOT tamper with the integrity of this product, it may cause harm the user otherwise. The power of final interpretation belongs to the manufacturer.